Mix-Pair-Share: Balancing Moments

Cambridge International AS & A Level Physics 9702  ·  Topic 4 (4.1)  ·  Cooperative activity

# How to run it (teacher)

- Learners mix around the room until the signal.

- On the signal, each learner pairs with the nearest person.

- Pose one prompt; partners solve it and share their reasoning and answer.

- Learners mix again and pair with someone new for the next prompt.

Skills: Communication and Critical Thinking. Each round, every learner explains a solution to a new partner, giving simultaneous interaction.

# Prompt set (reveal one at a time)

- P1. A door handle is fitted far from the hinges. Explain, using moments, why this makes the door easier to open.

- P2. A seesaw balances with a 200 N child 1.5 m from the pivot. Where must a 300 N child sit to balance it?

- P3. Explain why a longer spanner makes it easier to undo a tight nut.

- P4. A beam balances with a 5.0 N weight 0.60 m to the left of the pivot and an unknown weight 0.40 m to the right. Find the unknown weight.

- P5. A force of 12 N is applied at the end of a 0.25 m lever at 40 degrees to the lever. Find the moment about the pivot.

# Teacher answers

- P1. Placing the handle far from the hinges gives a large perpendicular distance, so the same push gives a larger moment and a greater turning effect, making the door easier to open.

- P2. 200 × 1.5 = 300 × d, so 300 = 300 d and d = 1.0 m from the pivot.

- P3. A longer spanner increases the perpendicular distance, so the same force produces a larger moment about the nut, making it easier to turn.

- P4. 5.0 × 0.60 = W × 0.40, so 3.0 = 0.40 W and W = 7.5 N.

- P5. Moment = 12 × 0.25 × sin 40° = 12 × 0.25 × 0.643 = 1.9 N m.