Worksheet answers and mark scheme: Moments

Cambridge International AS & A Level Physics 9702  ·  Topic 4 (4.1)  ·  Teacher copy

# Section A: Recall

- A1. The moment of a force about a point is the force multiplied by the perpendicular distance from the point to the line of action of the force; unit N m.

- A2. For a body in equilibrium, the sum of the clockwise moments about any point equals the sum of the anticlockwise moments about that point.

- A3. A couple is a pair of equal, opposite, parallel forces whose lines of action do not coincide. The torque of a couple is one of the forces multiplied by the perpendicular distance between them; unit N m.

# Section B: Moments

- B1. moment = 25 × 0.30 = 7.5 N m.

- B2. moment = 40 × 0.15 = 6.0 N m.

# Section C: Principle of moments

- C1. 250 × 1.2 = 300 × d, so 300 = 300 d and d = 1.0 m from the pivot.

- C2. The 2.0 N weight is 0.30 m from the pivot (50 cm − 20 cm). 2.0 × 0.30 = 3.0 × d, so 0.60 = 3.0 d and d = 0.20 m. The 3.0 N weight hangs 0.20 m from the centre on the other side, at the 70 cm mark.

# Section D: Couples

- D1. torque = one force × perpendicular distance = 6.0 × 0.40 = 2.4 N m.

# Section E: Reasoning and an angled force

- E1. moment = 15 × 0.20 × sin 50° = 15 × 0.20 × 0.766 = 2.3 N m.

- E2. Only the component of the force perpendicular to the arm produces a turning effect. Using the perpendicular distance (the distance along the arm multiplied by the sine of the angle) accounts for this; the full distance along the arm would overstate the moment.