Numbered Heads Together: Stress, Strain and the Young Modulus

Cambridge International AS & A Level Physics 9702  ·  Topic 6 (6.1)  ·  Cooperative activity

# How to run it (teacher)

- Form groups of four and have learners number off 1 to 4.

- Reveal one problem at a time. Groups discuss until every member can explain the answer.

- Call a random number; that learner answers for the group with no further help.

- Award a point for a correct, clearly explained answer. Aim for method and units, not just a value.

- Timing: about 18 minutes for the six problems. Skip Q6 if time is short; use it as a stretch.

Skills: Collaboration and Critical Thinking. Accountability comes from the random call after discussion.

# Team problem set

Useful relationships: σ = F / A,  ε = x / L,  E = σ / ε,  A = π d² / 4.

### Q1  Define

State what is meant by tensile stress and by tensile strain, and give the SI unit of each.

### Q2  Stress

A wire of cross-sectional area 2.0 × 10⁻⁷ m² supports a load of 60 N. Calculate the tensile stress.

### Q3  Strain

A wire of original length 1.50 m extends by 1.2 mm under load. Calculate the tensile strain.

### Q4  Young modulus

A metal wire of length 1.80 m and diameter 0.40 mm extends by 1.8 mm under a force of 25 N. Calculate the Young modulus.

### Q5  Graph

A stress-strain graph for a metal is a straight line from the origin to a point P, then curves. (i) Name point P. (ii) State how the Young modulus is found from the graph. (iii) What does the region beyond the elastic limit represent?

### Q6  Explain (stretch)

A learner claims: "A thick copper wire has a larger Young modulus than a thin copper wire of the same copper." Explain whether the learner is correct.

# Teacher answer key

### Q1

- Stress is the force per unit cross-sectional area, σ = F / A; unit pascal, Pa (N m⁻²).

- Strain is the extension per unit original length, ε = x / L; no unit (a ratio).

### Q2

σ = F / A = 60 / 2.0 × 10⁻⁷ = 3.0 × 10⁸ Pa (300 MPa).

### Q3

ε = x / L = 1.2 × 10⁻³ / 1.50 = 8.0 × 10⁻⁴.

### Q4

- A = π d² / 4 = π (0.40 × 10⁻³)² / 4 = 1.26 × 10⁻⁷ m².

- σ = F / A = 25 / 1.26 × 10⁻⁷ = 1.99 × 10⁸ Pa.

- ε = x / L = 1.8 × 10⁻³ / 1.80 = 1.0 × 10⁻³.

- E = σ / ε = 1.99 × 10⁸ / 1.0 × 10⁻³ = 2.0 × 10¹¹ Pa (200 GPa).

### Q5

- (i) P is the limit of proportionality.

- (ii) The Young modulus is the gradient of the straight-line region of the graph.

- (iii) The region beyond the elastic limit represents plastic (permanent) deformation: the wire does not return to its original length when unloaded.

### Q6

Incorrect. The Young modulus is a property of the material, not of the dimensions of the sample. The thicker wire has a larger cross-sectional area, so for the same force it has a smaller stress and stretches less (it is stiffer as a specimen), but the ratio stress / strain is the same for both because the material is the same.