Worksheet answers and mark scheme: Stress, Strain and the Young Modulus

Cambridge International AS & A Level Physics 9702  ·  Topic 6 (6.1)  ·  Teacher copy

# Section A: Recall

- A1. Stress is the force per unit cross-sectional area, σ = F / A; unit pascal, Pa (N m⁻²).

- A2. Strain is the extension divided by the original length, ε = x / L. It is a ratio of two lengths, so it has no unit.

- A3. E = stress / strain = σ / ε; unit pascal, Pa.

- A4. (a) False: strain has no unit. (b) True. (c) False: strength (breaking stress) and stiffness (the Young modulus) are different properties.

# Section B: Calculations

- B1. σ = F / A = 45 / 3.0 × 10⁻⁷ = 1.5 × 10⁸ Pa.

- B2. ε = x / L = 2.0 × 10⁻³ / 2.50 = 8.0 × 10⁻⁴.

B3.

- (a) A = π d² / 4 = π (0.60 × 10⁻³)² / 4 = 2.83 × 10⁻⁷ m².

- (b) σ = F / A = 30 / 2.83 × 10⁻⁷ = 1.06 × 10⁸ Pa.

- (c) ε = x / L = 1.0 × 10⁻³ / 1.60 = 6.25 × 10⁻⁴.

- (d) E = σ / ε = 1.06 × 10⁸ / 6.25 × 10⁻⁴ = 1.7 × 10¹¹ Pa (170 GPa).

B4.

- Rearrange E = F L / (A x) to give x = F L / (A E).

- x = (18 × 2.0) / (1.5 × 10⁻⁷ × 1.2 × 10¹¹) = 36 / (1.8 × 10⁴) = 2.0 × 10⁻³ m = 2.0 mm.

# Section C: Graph

- C1. (a) The limit of proportionality is the end of the straight-line region; the elastic limit is just beyond it, the last point from which the wire still returns to its original length.

- (b) Elastic behaviour up to the elastic limit; plastic behaviour beyond it.

- (c) E = gradient = 8.0 × 10⁸ / 4.0 × 10⁻³ = 2.0 × 10¹¹ Pa (200 GPa).

# Section D: Experiment

Marking points (one mark each, any six):

- measure original length L with a metre rule to a reference marker;

- measure diameter d with a micrometer at several points and directions, then average;

- calculate area A = π d² / 4;

- add known masses and record extension x for each load F = m g;

- plot stress against strain (or F against x);

- the Young modulus is the gradient of the linear region of the stress-strain graph (or gradient × L / A for an F against x graph);

- the diameter gives the largest uncertainty because A depends on d², so the percentage uncertainty in d is doubled;

- stay below the limit of proportionality.

# Section E: Challenge

- E1. (a) energy = ½ F x = ½ × 20 × 1.5 × 10⁻³ = 1.5 × 10⁻² J (0.015 J).

- (b) Assumption: the wire obeys Hooke's law (it is loaded within the limit of proportionality), so force is proportional to extension.

- (c) The energy equals the area under the force-extension graph. For a Hooke's law wire this area is a triangle, ½ × base × height = ½ x F.