CAROUSEL AND CARDS   ·   CAMBRIDGE IGCSE PHYSICS 0625   ·   1.1 MEASUREMENT

# Measuring length and volume: station cards

Print the station cards and place one at each station. Set up duplicate sets so groups stay small. Pairs rotate on a signal and record every value with its unit on the record sheet overleaf. Any learner may be called to report a reading.

STATION 1 · LENGTH (RULE AND STRING) Measure the length of the block, and the curved edge using string then the rule. Read square to the scale, to the nearest millimetre. Record each value with its unit.

STATION 2 · VOLUME OF A LIQUID (MEASURING CYLINDER) Measure the volume of liquid A and of liquid B. Read the bottom of the meniscus, with your eye level with the liquid. Record in cm³.

STATION 3 · VOLUME OF A STONE (DISPLACEMENT) Read the water level, lower the stone in gently, then read the level again. The volume of the stone is the rise: final reading minus first reading. Keep it fully submerged, with no trapped air.

## Carousel record sheet

Names: ________________________	Class: __________

| Measurement | Value | Unit | How I read it |
| --- | --- | --- | --- |
| Station 1: length of the block |  | cm or mm |  |
| Station 1: curved edge (string then rule) |  | cm |  |
| Station 2: volume of liquid A |  | cm³ |  |
| Station 2: volume of liquid B |  | cm³ |  |
| Station 3: water level before |  | cm³ |  |
| Station 3: water level after |  | cm³ |  |
| Station 3: volume of stone (after − before) |  | cm³ |  |

Compare and reconcile Swap your sheet with another pair. Where do your readings differ, and why? Use the ideas of smallest division, parallax, and reading the bottom of the meniscus.

Original work by the TheLucidSTEM team. Designed for the lesson on this site; no past paper material is reproduced.